Junkyard Drawers!

Good morning team,

It seems that freshmen have been disappearing down by the teacher’s room at the end of the hall.  No one knows how or why, but some speculate that they are being held prisoner by a stir-crazy Mr. Larson, who has returned from retirement to force them to learn Algebra.

Your mission, and you will choose to accept it, is to build and test a reconnaissance vehicle.  Eventually, it may be fitted with a fiber optic video surveillance device, but for now it must simply be a battery-powered vehicle that can travel 10 meters, stop, and (if possible) change direction and come back so that the data might be extracted.

Of course it may be dark down the hallway, so your vehicle should be outfitted with headlights (you may provide your own or use mine).  And because we may use these to accomplish other missions, you might want to make them as light as possible.

Considerations:

Power source:  What type?  How many?  Which have too much voltage?  Not enough?  AAAs?  Watch batteries?  9V? 6V?  Which will most efficiently run your motor?

Switch:  How will your car start?  How will it stop?  There are a lot of types of switches that can break the circuit and stop the car.  Which type will you use?  Knife switch?  Toggle switch?  Key switch?  Reed switch?  Rocker Switch?  Push-button switch?

Motor: What kind?  Where will you find one?  How big should it be?  How will you mount it to your wheels?  How will you mount it to your car?

Wheels and gears:  What kind?  How big?  Will they slow your vehicle down?  Do you want that?  Gear ratio? What is that?

Resistors:  Do you need to increase resistance?  You can buy resistors at Radio Shack (in Leominster Twin City plaza) extremely cheaply.

Headlights: How will you wire them? In series?  Parallel?  Is there a benefit to both?

How would you wire the car to maximize efficiency?  You don’t want to blow out the headlights, so you may need to cut down the current.  You will need a schematic diagram of the electric circuit that you create.

Heat! If you use materials that have high resistances, you will produce a lot of heat!  Be careful not to touch components until you are sure they are not hot!

You will need to bring in the materials needed to assemble these all this week.  We will begin assembly tomorrow.  You will probably have about a week to build and practice with your vehicles.  You may bring in any tools or materials that you think you will need in order to accomplish this project.

Keep a journal of your progress to be handed in at the end of the project. ALL ideas, not just what works.  Hire someone to be the recorder.

Remember, not only should it work, it should also look really cool… and maybe scientific. Or science-fiction-ish.

Good Luck.  Have fun,

-Mr. Cranson